Beak & Skiff

1. Automated Counting and Filling Systems

A) Optical Sensors: - Use optical or laser counting sensors integrated with a conveyor system to count gummies accurately as they pass through.

Function: Converts light signals into electrical signals, allowing the machine to detect the presence and position of gummy candies.

Application: Primarily used for counting gummies during packaging, ensuring the correct number of gummies are placed in each package.

Sensor type: Often a through-beam photoelectric sensor, where a light emitter sends a beam towards a receiver, and the interruption of the beam by a gummy trigger a signal.

B) Vibratory Feeders: Automate the process using vibratory feeding systems that count and separate gummies into precise portions.

Function: 
1) Feeding:

- Vibratory feeders use vibration to move gummies in a controlled manner along a tray or track. This ensures smooth transportation of gummies without clogging or damage.

2) Counting:

- Integrated counting systems (optical or vision-based) ensure that each portion contains the exact number of gummies required.

3) Separation:

- The vibratory motion aligns and separates the gummies, preventing them from clumping or overlapping. This is critical for accurate packaging and portioning.

4) Packaging:

- The separated and counted gummies are directed into the packaging unit, which could include form-fill-seal machines or other automated packaging systems.

Application: Automated sorting, counting, and portioning of gummies for precise packaging. This ensures consistent weight or quantity in each package, adhering to quality control and regulatory standards.

Automation Provider:

sales@gummygenix.com

Phone +1 941.866.6187 (US)​

Automated Filling Machines: Machines with pre-programmed settings can ensure that exactly 10 gummies are dispensed into each bag.

Examples: Multi-head weighers or counting machines tailored for gummy sizes.

C) Weight and Count: Explore "weigh and count" technology as weight is often proportional to the number of gummies.